Technical specification LBAi W GATE Single

Tender documents:

The ultra-fast folding gate shall be reliable and robust, designed for intensive, continuous operation with a 4 m clear opening.

It shall comply with standard 13241-1 and be validated by CSTB, with installation requiring no additional safety photocells or safety edges. It shall meet the following specifications:

Technical specifications

- Custom height from 2 to 3 m

- Single: 2 leaves, 3 to 5 m

- 6 mm thick steel pivot post – 350 x 250 mm cross-section

- Anti-corrosion e-coat treatment

- Powder coating baked at 250°, with a choice of RAL colours: White 9010, Grey 7016, Green 6005, Red 3020 or Black 9005

- Aluminium leaves with 30 x 30 mm bars and variable spacing suited to the gate length, not exceeding 110 mm

- Concealed drive integrated into the pivot post, accessible through a dedicated door below the control compartment door

- Control system accessible through two doors integrated into the pivot post, with access when the gate is open or closed.

- (exposed motors and/or separate control cabinets shall not be permitted)

Equipment

- Reversible geared motor

- Brake motor enabling the gate to be locked in the event of a power failure

- Manual release from the motor compartment

- Opening speed of 1.2 m/s

- Force limiter in the event of impact

- 2 sets of IR photocells with transmitter/receiver

- Contactless inductive sensors for position control

- Held in the open or closed position

- 90° opening

- Access doors fitted with anti-vandalism compression locks

- LED area lighting

- 500 kg electromagnetic lock

- 2 anchoring templates with anchor rods

- Lifting eyes for easy, safe positioning by installers

Integrated ONE-C control board

- Hardwired or network cable control; several communication protocols (Modbus TCP/IP, RS485)

- Centralised communication interface (LED display)

- Remote configuration and control via web server

- Variable-frequency motor control to adjust:

- Acceleration and braking ramps

- Opening and closing speeds

- SD card updates with a single program

- Plug-in terminal block

LBAi functions

- Integrated detection without ground loops or additional infrared photocells

- No detection-related civil works and reduced wiring requirements

- Recognition of 7 user categories: pedestrians, bicycles, motorcycles, cars, vans, trucks and buses

- Up to 5 configurable detection zones

- Range up to 10 m

- 170° horizontal and 70° vertical coverage

- Precise settings to limit unwanted detections

- Configurable access permissions by category

- Management of access authorisations and denials

- Customisable time schedules

- Configuration via a secure local Wi-Fi connection, independent of the site's network

- Continuous day and night operation with reliable performance in adverse weather

- Activity log available to assist diagnostics

- Simple updates via SD card

- Planned upgrades: vehicle counting and future recognition of specific vehicles

Options to include in the tender documents if selected

- 115° opening

- Alternative colour to be selected from the RAL chart

- Perforated aluminium infill panel with an LBA Design pattern

- Ø60 mm LED light mounted on the receiver post

- Controlled red/green LED strip

- Additional 500 kg electromagnetic lock

Compliance

- Compliant with standard 13241-1

- Validation by CSTB (French Scientific and Technical Centre for Building) without additional safety edges or photocells