Technical specification LBAi 10

Tender documents:

The automatic barrier shall be robust and designed for intensive, continuous operation. It shall be a LA BARRIÈRE AUTOMATIQUE LBA 10. The barriers shall accommodate straight fibreglass booms and meet the following specifications:

The barrier shall incorporate LBAi intelligent detection, user classification and access rule management functions.

Technical specifications

- Housing and door made from 3 mm thick DKP steel sheet with an e-coat finish.

- Cover made from 2 mm thick aluminium sheet with a phosphate treatment.

- 10,000 operations/day

- Variable operating time from 2 to 7 seconds

- Three-phase motor and 230 V single-phase power supply

Equipment

- Axial Ø140 mm reinforced fibreglass boom, up to 10 m in one piece without guy wires. This composite boom shall provide greater flexibility in the event of impact. It is particularly suitable for managing heavy goods vehicle traffic.

- Reinforced mounting bracket

- Barrier housing with a self-locking or reversible geared motor

- Counterbalance with a compression spring (torsion or tension springs are too fragile)

- Contactless inductive sensors for barrier control.

- Held in the open or closed position

Integrated ONE-C control board

    - Hardwired or network cable control; several communication protocols (Modbus TCP/IP, RS485)

    - Centralised communication interface (LED display)

    - Certified PL-d, Category 2, under EN ISO 13849-1

    - Remote configuration and control via web server

    - Variable-frequency motor control to adjust:

    - Acceleration and braking ramps

    - Opening and closing speeds

    - SD card updates with a single program

    - Plug-in terminal block

LBAi functions

- Integrated detection without ground loops or additional infrared photocells

- No detection-related civil works and reduced wiring requirements

- Recognition of 7 user categories:

- Pedestrians, bicycles, motorcycles, cars, vans, trucks and buses

- Up to 5 configurable detection zones

- Range up to 10 m

- 170° horizontal and 70° vertical coverage

- Precise settings to limit unwanted detections

- Configurable access permissions by category

- Management of access authorisations and denials

- Customisable time schedules

- Configuration via a secure local Wi-Fi connection, independent of the site's network

- Continuous day and night operation with reliable performance in adverse weather

- Activity log available to assist diagnostics

- Simple updates via SD card

- Planned upgrades: vehicle counting and future recognition of specific vehicles

Options to include in the tender documents if selected

- Impact-protection rotation mechanism

- Manual rotation of the housing shall prevent breakage if a vehicle strikes or forces the barrier.

- After an impact, the boom shall be repositioned manually without tools or any specific work inside the housing.

- The system shall operate by friction on the anchoring template, ensuring that the boom moves no further than the vehicle pushes it. (No ball bearings or repositioning pins requiring a technician's intervention.)

- Choice of RAL colour

- Automatic raising in the event of a power failure

- Boom support with rubber damper

- Boom support with electromagnetic lock

- Internal locking mechanism preventing manual raising

- Anti-vandalism package (reinforcing band, bolt covers and accessory guards)

- Signs, audible and visual signalling

- LED cover (red/green)

Technical data

- MCBF (mean cycles between failures): 10,000,000 cycles

- MTBF (mean time between failures): 15,000 hours

- Daily operating cycles: 10,000 operations per day

- MTTR (mean time to repair): 60 minutes

- Ingress protection rating: IP54