### Technical specification LBAi 86

Tender documents:

The automatic barrier shall be robust and designed for intensive, continuous operation in high-security environments.

It shall be a LA BARRIÈRE AUTOMATIQUE LBAi 86.

The barrier shall incorporate LBAi intelligent 
      detection, user classification and access rule management functions.

Technical specifications:

- Housing and door made from 3 mm thick DKP steel sheet with an e-coat finish

- Cover made from 2 mm thick aluminium sheet with a phosphate treatment

- Intensive, continuous operation: 10,000 operations/day

- Variable-speed opening/closing, from 2 seconds depending on boom length and configuration

- Three-phase motor and 230 V single-phase power supply

Equipment:

- Reinforced axial boom:

    - 180 x 118 mm straight boom: up to 6 m

    - 135 x 93 mm oval boom: up to 8 m

- Reinforced mounting bracket

- Barrier housing with a reversible geared motor

- Counterbalance with a compression spring

- Integrated anti-vandalism and anti-intrusion package

- Angular sensor indicating boom position to within one degree

- Held in the open or closed position

- Set of keys for the door lock

- Integrated ONE-C control board:

    - Hardwired or network cable control; Modbus TCP/IP and RS485 protocols

    - Certified PL-d, Category 2, under EN ISO 13849-1

    - Centralised communication interface with LED display

    - Remote configuration and control via web server

    - Variable-frequency control for adjusting acceleration and braking ramps

    - Adjustable opening and closing speeds

    - SD card updates with a single program

    - Plug-in terminal block

LBAi functions:

- Integrated detection without ground loops or additional infrared photocells

- No detection-related civil works and reduced wiring requirements

- Recognition of 7 user categories:

    - Pedestrians, bicycles, motorcycles, cars, vans, trucks and buses

- Up to 5 configurable detection zones

- Range up to 10 m

- 170° horizontal and 70° vertical coverage

- Precise settings to limit unwanted detections

- Configurable access permissions by category

- Management of access authorisations and denials

- Customisable time schedules

- Configuration via a secure local Wi-Fi connection, independent of the site's network

- Continuous day and night operation with reliable performance in adverse weather

- Activity log available to assist diagnostics

- Simple updates via SD card

- Planned upgrades: vehicle counting and future recognition of specific vehicles

Options to include:

- Choice of RAL colour

- Boom support with rubber damper

- Automatic raising upon loss of power, depending on configuration

Technical data:

- MCBF (mean cycles between failures): 10,000,000 cycles

- MTBF (mean time between failures): 15,000 hours

- Operating temperature: -30°C to +55°C

- Ingress protection rating: IP54