Technical specification LBAi 74

Tender documents:

The automatic barrier shall be robust and designed for intensive, continuous operation. It shall be a LA BARRIÈRE AUTOMATIQUE LBA 74. The barriers shall accommodate straight or articulated oval aluminium booms and meet the following specifications:

The barrier shall incorporate LBAi intelligent detection, user classification and access rule management functions.

Technical specifications

- Housing and door made from 2 mm thick DKP steel sheet with an e-coat finish

- Cover made from 2 mm thick aluminium sheet with a phosphate treatment

- 10,000 operations/day

- Opening/closing time from 2.5 to 3.8 seconds

- Three-phase motor and 230 V single-phase power supply

Equipment

- Axial boom with reinforced mounting bracket

- Secure, robust axial boom mounting bracket with a reinforced base and non-removable construction

- 135 x 93 mm semi-reinforced straight boom up to 3.50 m

- 135 x 93 mm reinforced articulated aluminium boom up to 3.50 m

- Automatic reopening upon loss of power

- Barrier housing with geared motor, counterbalance and axial mounting bracket.

- Compression-spring counterbalance (tension or torsion springs prohibited for reliability reasons)

- Angular sensor indicating boom position to within one degree

- Set of keys for the door lock

- Instructions for assembly, connections, commissioning and maintenance.

Integrated ONE-C control board

    - Hardwired or network cable control; several communication protocols (Modbus TCP/IP, RS485)

    - Certified PL-d, Category 2, under EN ISO 13849-1

    - Centralised communication interface (LED display)

    - Remote configuration and control via web server

    - Variable-frequency motor control to adjust:

    - Acceleration and braking ramps

    - Opening and closing speeds

    - SD card updates with a single program

    - Plug-in terminal block

LBAi functions

- Integrated detection without ground loops or additional infrared photocells

- No detection-related civil works and reduced wiring requirements

- Recognition of 7 user categories:

- Pedestrians, bicycles, motorcycles, cars, vans, trucks and buses

- Up to 5 configurable detection zones

- Range up to 10 m

- 170° horizontal and 70° vertical coverage

- Precise settings to limit unwanted detections

- Configurable access permissions by category

- Management of access authorisations and denials

- Customisable time schedules

- Configuration via a secure local Wi-Fi connection, independent of the site's network

- Continuous day and night operation with reliable performance in adverse weather

- Activity log available to assist diagnostics

- Simple updates via SD card

- Planned upgrades: vehicle counting and future recognition of specific vehicles

Options to include in the tender documents if selected

- Choice of RAL colour

- Aluminium housing

- LED cover

- Automatic reopening upon loss of power

- Internal anti-tamper locking device

- Boom support with rubber damper (excluding breakaway or swing-away boom options)

- Boom support with electromagnetic lock (excluding breakaway or swing-away boom options)

- Boom support with electromechanical lock (excluding breakaway or swing-away boom options)

- Flashing LED lights

- Signs, audible and visual signalling

Technical data

- MCBF (mean cycles between failures): 5,000,000 cycles

- MTBF (mean time between failures): 15,000 hours

- Daily operating cycles: 10,000 operations per day

- MTTR (mean time to repair): less than 30 minutes

- Ingress protection rating: IP54