Technical specification LBAi 63PK

Tender documents:

The automatic barrier shall be robust and designed for intensive, continuous car park operation. It shall be a LA BARRIÈRE AUTOMATIQUE LBA 63PK. The barriers shall accommodate straight or articulated oval aluminium booms and meet the following specifications:

The barrier shall incorporate LBAi intelligent detection, user classification and access rule management functions.

Technical specifications

- Housing and door made from 2 mm thick DKP steel sheet with an e-coat finish

- Cover made from 2 mm thick aluminium sheet with a phosphate treatment

- 15,000 operations/day

- Opening/closing time from 0.8 to 2.4 seconds

- Three-phase motor and 230 V single-phase power supply

Equipment

- 84 x 57 mm straight or universal articulated oval aluminium boom with reflective strips, up to 3.80 m

- The articulated boom shall require no advance measurements by the installer. It shall be fitted on site regardless of the available ceiling clearance.

- Breakaway boom secured by a stainless steel screw and nylon nut

- Breakaway boom detection

- Automatic reopening upon loss of power

- Barrier housing with a reversible geared motor

- Compression-spring counterbalance (tension or torsion springs prohibited for reliability reasons)

- Angular sensor indicating boom position to within one degree.

- Held in the open or closed position

- Set of keys for the door lock.

Integrated ONE-C control board

    - Hardwired or network cable control; several communication protocols (Modbus TCP/IP, RS485)

    - Certified PL-d, Category 2, under EN ISO 13849-1

    - Centralised communication interface (LED display)

    - Remote configuration and control via web server

    - Variable-frequency motor control to adjust:

    - Acceleration and braking ramps

    - Opening and closing speeds

    - SD card updates with a single program

    - Plug-in terminal block

LBAi functions

- Integrated detection without ground loops or additional infrared photocells

- No detection-related civil works and reduced wiring requirements

- Recognition of 7 user categories:

- Pedestrians, bicycles, motorcycles, cars, vans, trucks and buses

- Up to 5 configurable detection zones

- Range up to 10 m

- 170° horizontal and 70° vertical coverage

- Precise settings to limit unwanted detections

- Configurable access permissions by category

- Management of access authorisations and denials

- Customisable time schedules

- Configuration via a secure local Wi-Fi connection, independent of the site's network

- Continuous day and night operation with reliable performance in adverse weather

- Activity log available to assist diagnostics

- Simple updates via SD card

- Planned upgrades: vehicle counting and future recognition of specific vehicles

Options to include in the tender documents if selected

- Choice of RAL colour

- Aluminium housing

- Rear swing-away mechanism

- LED cover

- Illuminated boom

- Foam-padded boom

- Forced-entry attempt alarm

- Internal anti-tamper locking device

- Boom support with rubber damper (excluding breakaway or swing-away boom options)

- Boom support with electromagnetic lock (excluding breakaway or swing-away boom options)

- Boom support with electromechanical lock (excluding breakaway or swing-away boom options)

Technical data

- MCBF (mean cycles between failures): 10,000,000 cycles

- MTBF (mean time between failures): 15,000 hours

- Daily operating cycles: 15,000 operations per day

- MTTR (mean time to repair): less than 15 minutes

- Ingress protection rating: IP54